| ASSISTIVE AI & COMPUTER VISION RESEARCH PROJECT APEC SMART GLASSES FOR THE BLIND Real-Time Multimodal Vision AI, Local Vietnamese ASR & Tactile Accessibility | Repository: thanh1912-ut/APEC_mat_kinh... Target SHA: d9ba1e0 (main branch) Current Release: Android Pilot v0.4.0 (Code 8) Speech Engine: Zipformer RNN-T (sherpa-onnx) Vision AI: Google Gemini 3.8 Flash |
| --- | --- |

| < 180 ms Local STT Latency | 100% Tactile Hardware Volume Keys | Zero Audio Leak Speech Kept On-Premises | 651 Test Cases 322 Python + 329 Jest |
| --- | --- | --- | --- |

| 1. PROJECT MISSION, GOALS & SCOPE APEC Smart Glasses is an assistive engineering project dedicated to providing blind and visually impaired users with independent spatial awareness, instant object recognition, and interactive guidance for domestic appliances. ■ True Non-Visual Accessibility: Eliminates touchscreen barriers by utilizing physical smartphone volume keys (Volume Up = Listen/Capture/Send; Volume Down = Cancel/Stop) paired with audio earcons. ■ Absolute Speech Privacy: Streaming speech recognition executes entirely on-premises using lightweight Zipformer RNN-T models (~51MB ONNX). Raw user speech is never leaked to cloud APIs. ■ Appliance Guidance Engine (v0.4.0): Provides verified step-by-step assistance to locate and trigger physical controls on air conditioners, TV remotes, and household appliances. ■ Spatial Vision Roadmap: Pre-engineered abstraction layer designed to integrate STMicroelectronics VL53L9CX Direct ToF LiDAR sensors for 3D millimeter-level hazard detection. 2. SYSTEM ARCHITECTURE & DATA FLOW [A. Mobile Client]: React Native/Expo app captures rear camera JPEG (max 1280px) and voice query (M4A/AAC). [B. FastAPI Gateway]: Authenticates SHA-256 Bearer tokens, enforces 4MiB limits, queues into WorkGate. [C. Local ASR Worker]: PyAV resamples audio to mono 16kHz; sherpa-onnx transcribes Vietnamese text in RAM. [D. Vision AI]: Image and transcript queried to Gemini 3.8 Flash / 3.1 Flash Lite with store=false. [E. Audio Delivery]: Structured JSON response spoken via Android TTS and earcons without TalkBack clash. Component; Technology & Specification. Mobile Client; React Native, Expo SDK 57, TypeScript. Backend Gateway; Python 3.11, FastAPI, PyAV, WorkGate. Vietnamese STT; sherpa-onnx (Zipformer RNN-T 51MB). Multimodal Vision; Google Gemini 3.8 Flash (store=false). Guidance State; PostgreSQL + Redis (CAS Locks). | 3. TECHNICAL INNOVATIONS & FEATURES ■ [UX] Tactile Hardware Control: Single short-press Volume Up activates audio listening; second press captures and uploads. Short-press Volume Down stops TTS readout immediately or cancels pending requests. Long press passes through to Android system volume. ■ [PRIVACY] On-Premises Vietnamese ASR: Integrates hynt/Zipformer-30M-RNNT-Streaming-6000h via sherpa-onnx. Audio chunks are decoded in ephemeral RAM and discarded upon turn completion, guaranteeing zero voice surveillance. ■ [AI AGENT] Verified Remote Guidance: Unlike standard chat models that hallucinate button coordinates, the guidance engine in v0.4.0 enforces strict verification checkpoints, requiring visual proof before instructing the user to press a button. ■ [HARDWARE] ToF LiDAR Sensor Extension: Pre-configured integration for STMicroelectronics VL53L9CX Direct Time-of-Flight sensor with a 54x44 matrix (2,376 distance zones) for millimeter-level obstacle distance detection. 4. ENGINEERING RIGOR & VISUAL PROOF Verification Suite: 322 automated Python pytest cases (WebRTC, tokens, WorkGate) + 329 Jest mobile tests across 17 suites with zero regressions. Strict zero-secret policy in APK. Assistant UI; Settings UI; VL53L9CX ToF LiDAR. Security Guardrails: Bearer tokens stored in Expo SecureStore validated via SHA-256 hashes. Ephemeral memory buffers prevent data persistence. In-memory WorkGate limits single active task per token.. |
| --- | --- |

APEC Assistive Technologies Project • Open-Source Engineering for the Visually Impaired   |   Target Commit: d9ba1e0 • Research & Open Pilot • A4 Poster Document